- Arduino Uno

- Adafruit Motor Shield (AFMotor library)

- 5 IR sensor array

- 4 DC motors (4WD robot chassis)

Here is the wiring diagram in text form:

5 IR SENSOR ARRAY
   -----------------------------
   FAR LEFT   -> A4
   INNER LEFT -> A3
   CENTER     -> A2
   INNER RIGHT-> A1
   FAR RIGHT  -> A0

   VCC -> 5V (Arduino)
   GND -> GND (Arduino)

Motor Shield connections (AFMotor library motor numbers):

MotorFL (Front Left)  -> M3
MotorFR (Front Right) -> M4

MotorRL (Rear Left)   -> M2
MotorRR (Rear Right)  -> M1

Robot layout:

FRONT

   M3               M4
Front Left      Front Right
 Motor             Motor


 A4 A3 A2 A1 A0
[IR][IR][IR][IR][IR]
(FarL InL Ctr InR FarR)


Rear Left       Rear Right
 Motor             Motor
  M2                M1

         BACK

Power:

Battery + -> Motor Shield EXT_PWR (+)
Battery - -> Motor Shield GND

Arduino powered by:
USB
OR
Battery through VIN

IR sensor wiring:

| IR Sensor | Arduino Pin |
| --- | --- |
| Far Left | A4 |
| Inner Left | A3 |
| Center | A2 |
| Inner Right | A1 |
| Far Right | A0 |
| VCC | 5V |
| GND | GND |

Motor mapping:

| Motor Position | Shield Port |
| --- | --- |
| Front Left | M3 |
| Front Right | M4 |
| Rear Left | M2 |
| Rear Right | M1 |

| Front Right | M4 |
| --- | --- |
| Rear Left | M2 |
| Rear Right | M1 |